INTRODUCTION

In digital communication, forward error-correcting (FEC)codes play a pivotal role in mitigating random errors at thetransmitter [1].Understanding FEC encoders at the receivingend is crucial for decoding. While some receiver systemspossess prior knowledge of the encoder used at the transmitter,enabling successful decoding, situations arise whereblind estimation of the channelencoder becomes imperative,particularly when the receiver lacks prior knowledge [2]. Consequently,various innovative techniques and algorithms for theblind reconstruction of channel encoders have been introduced.However, these advancements pose new challenges. Addressingthese challenges have significant implications, especiallyin non-cooperative communication scenarios, where accuratereconstruction of the channel encoder is vital for successfullydecoding the signals from unknown sources. Blind recognitionof channel encoder parameters can enhance spectral efficiencyby conserving channel resources [1], [2]. The blind recoveryof convolutional encoders has been explored in prior research,as seen in works such as [2] and [3], leveraging algebraicand dual-code properties. Studies in [4] and [5] delved intothe blind reconstruction of a parallel concatenation of tworecursive systematic convolutional (RSC) encoders using iterativeexpectation-maximization (EM) and the least-squaremethod. Methods in [6] were based on the rank deficiency ofthe data matrix for recognizing encoders parameters