SYSTEM ANALYSIS

### Existing System:

The existing methods for recognizing and classifying channel encoders, particularly forward error-correcting (FEC) codes, typically rely on traditional algorithmic approaches. These include algebraic methods, rank deficiency techniques, or dual-code approaches to estimate encoder parameters. Some notable methods include blind recognition techniques that are highly sensitive to noise and computational complexity, especially at low signal-to-noise ratios (SNR). Mathematical models used in these methods often fail to capture complex patterns, making them unsuitable for real-time, noisy environments.

Disadvantages of the Existing System:

- High computational complexity: Most traditional methods are computationally intensive, especially in noisy environments, which makes real-time execution challenging.

- Noise sensitivity: These methods tend to perform poorly at low SNR levels, reducing their effectiveness in practical communication systems.

- Limited scalability: Traditional techniques are often designed for specific encoder types, making them less versatile when dealing with different FEC codes.

- Inability to handle large data volumes: Existing approaches are inefficient when handling large datasets, leading to slower processing and lower accuracy.

- Complex implementation: The use of algebraic or rank-based models makes the system difficult to implement and adapt to new technologies, especially in non-cooperative environments.

### Existing System Algorithm:

- Input:

  - Information bits sent through the communication channel.

  - Type of channel encoder is unknown.

- Step 1: Preprocessing

  - Apply algebraic techniques or rank-based methods to preprocess the encoded data.

  - In some cases, dual-code properties or Hamming weight distributions may be used to estimate parameters of the encoder.

- Step 2: Blind Estimation

  - Use methods such as least-square techniques or expectation-maximization (EM) algorithms to estimate encoder parameters blindly.

  - Alternatively, apply a mathematical model that uses statistical distributions of errors and code properties to reconstruct the encoder.

- Step 3: Decoder Selection

  - Based on the estimated parameters from step 2, select a possible decoder (e.g., block, convolutional, BCH).

  - Use algebraic models to match the encoding scheme with known decoding models.

- Step 4: Decoding

  - Apply the selected decoder to the received information.

  - If the decoding is successful, classify the encoder; otherwise, repeat the parameter estimation.

- Step 5: Output

  - Successfully decode the message and classify the encoder type.

### Proposed System:

The proposed system leverages a deep learning-based approach using Convolutional Neural Networks (CNN) to classify four distinct types of channel encoders: block encoders, convolutional encoders, Bose-Chaudhuri-Hocquenghem (BCH) encoders, and polar encoders. This CNN-based approach aims to automate the recognition of encoders in a more robust, accurate, and scalable manner. The model is designed to handle input data from non-cooperative communication systems, where encoder parameters are unknown. With the use of convolutional layers for feature extraction, the model identifies patterns within the encoded data and achieves high classification accuracy.

Advantages of the Proposed System:

- Improved accuracy: The CNN model achieves over 95% classification accuracy even in low bit-error-rate (BER) environments, significantly outperforming traditional methods.

- Noise resilience: Unlike traditional models, the proposed system is robust in handling noisy transmission channels, particularly with input data sizes that improve accuracy as they increase.

- Scalability and adaptability: The CNN-based approach can easily scale to classify various types of encoders, making it adaptable to new communication standards like 5G.

- Real-time processing: With optimized architectures, the deep learning model processes large data volumes efficiently, making it suitable for real-time communication systems.

- Automation and flexibility: The model can automatically recognize and classify encoders without requiring prior knowledge of the channel conditions, making it ideal for non-cooperative communication scenarios.

### Proposed System Algorithm (Using CNN):

- Input:

  - Encoded data samples from the communication channel (bitstream).

- Step 1: Preprocessing

  - Use MATLAB or similar tools to generate datasets for four different encoding schemes: block (Hamming), convolutional, BCH, and polar codes.

  - Simulate an Additive White Gaussian Noise (AWGN) channel and perform demodulation of the received signal (e.g., BPSK modulation).

- Step 2: CNN Model Initialization

  - Design a Convolutional Neural Network (CNN) with layers for convolution, max pooling, and fully connected operations (as described in [13†source]).

  - Use appropriate activation functions like ReLU for intermediate layers and softmax for the final output layer to classify the encoders.

- Step 3: Training

  - Use a labeled dataset containing samples of encoded data generated using different encoders.

  - Train the CNN model using a cross-entropy loss function and an optimizer such as Adam.

  - Monitor training progress and adjust hyperparameters like learning rate, batch size, and dropout.

- Step 4: Testing

  - Evaluate the model's classification accuracy using a dedicated test set (e.g., using a dataset of various lengths and BER values).

  - Test classification across various bit-error rates (BER) and sample sizes (e.g., sizes 512, 1024, 2048).

- Step 5: Classification

  - Use the trained CNN model to classify the input data as one of the four types of encoders: block, convolutional, BCH, or polar.

  - Compute accuracy metrics (true positives, false positives) for each encoder category.

- Step 6: Output

  - Output the classification result with the corresponding encoder type.

  - Optionally, display performance metrics such as confusion matrix and classification accuracy for different input sizes and BER values.

Feel free to expand the remaining parts of the Existing System and Proposed System sections with additional paragraphs based on the insights from the project.

SYSTEM REQUIREMENTS:

HARDWARE REQUIREMENTS:

- System			: 	Intel Core i7.

- Hard Disk 			: 	1TB.

- Monitor			: 	15’’ LED

- Input Devices		: 	Keyboard, Mouse

- Ram				: 	8GB.

SOFTWARE REQUIREMENTS:

- Operating system 		: 	Windows 11.

- Coding Language		:	Python

- Tool				:	PyCharm, Visual Studio Code

- Database			:	SQLite

REFERENCES:

Page(s): 1 - 14

Date of Publication: 02 June 2023

ISSN Information:

Print ISSN: 1524-9050

Electronic ISSN: 1558-0016

DOI: 10.1109/TITS.2023.3278533

Publisher: IEEE

Funding Agency:

10.13039/501100001659-Deutsche Forschungsgemeinschaft (Grant Number: 393541319/GRK2475/1-2019 and 146371743/TRR)

German Federal Ministry of Education and Research (Grant Number: 13N15319)